## SIMATS ENGINEERING

### ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

QUESTIONS: 5	Total Marks:50

Annexure A

# Scenario-Based

Code of Conduct:

I NIVEDITHA/192421440L (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: NIVEDITHA

Annexure A

# Scenario-Based

- A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25 School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

  - Determine	the	network	address	and	broadcast	address	for	each department.

  - Identify the valid host IP address range for each subnet.

  - Calculate the number of usable host addresses available in each subnet.

  - Determine the remaining unused IP address range within the given network block.

ANS: Smart Campus – IPv4 Subnetting

Given IP block: 192.168.100.0/24

1. School of Computing – /25

- Network address: 192.168.100.0

- Subnet mask: 255.255.255.128

- Broadcast address: 192.168.100.127

- Valid host range: 192.168.100.1 – 192.168.100.126

- Usable hosts: \(2^7-2=126\)

- This subnet supports computers, servers, and other devices in the department.

2. School of Electronics – /26

- Network address: 192.168.100.128

- Subnet mask: 255.255.255.192

- Broadcast address: 192.168.100.191

- Valid host range: 192.168.100.129 – 192.168.100.190

- Usable hosts: \(2^6-2=62\)

- This subnet provides IP addresses for laboratory computers and electronic devices.

3. School of Mechanical Engineering – /27

- Network address: 192.168.100.192

- Subnet mask: 255.255.255.224

- Broadcast address: 192.168.100.223

- Valid host range: 192.168.100.193 – 192.168.100.222

- Usable hosts: \(2^5-2=30\)

- This subnet supports computers and devices in the mechanical department.

4. Remaining unused IP addresses

- Unused range: 192.168.100.224 – 192.168.100.255

- Total unused addresses: 32.

- These addresses can be allocated to new departments or additional devices in the future.

Conclusion: Subnetting divides the campus network into smaller networks, uses IP addresses efficiently, and helps manage each department separately.

- A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24 Quality Assurance → /25 Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

  - Determine the network address and broadcast address for each department.

  - Identify the valid range of host IP addresses.

  - Calculate the number of usable hosts in each subnet.

  - Identify the unused IP address range remaining within the allocated IP block.

ANS: Software Company – IPv4 Subnetting

Given IP block: 10.10.0.0/16

1. Software Development – /24

- Network address: 10.10.0.0

- Subnet mask: 255.255.255.0

- Broadcast address: 10.10.0.255

- Valid host range: 10.10.0.1 – 10.10.0.254

- Usable hosts: \(2^8-2=254\)

- This subnet supports a large number of development computers.

2. Quality Assurance – /25

- Network address: 10.10.1.0

- Subnet mask: 255.255.255.128

- Broadcast address: 10.10.1.127

- Valid host range: 10.10.1.1 – 10.10.1.126

- Usable hosts: \(2^7-2=126\)

- This subnet is used for testing systems and applications.

3. Human Resources – /26

- Network address: 10.10.1.128

- Subnet mask: 255.255.255.192

- Broadcast address: 10.10.1.191

- Valid host range: 10.10.1.129 – 10.10.1.190

- Usable hosts: \(2^6-2=62\)

- This subnet supports HR computers and office devices.

4. Executive Management – /27

- Network address: 10.10.1.192

- Subnet mask: 255.255.255.224

- Broadcast address: 10.10.1.223

- Valid host range: 10.10.1.193 – 10.10.1.222

- Usable hosts: \(2^5-2=30\)

- This subnet supports management computers and devices.

5. Remaining unused IP addresses

- Unused range: 10.10.1.224 – 10.10.255.255

- These addresses remain available for future network expansion.

- The company can create additional subnets when new departments are added.

Conclusion: Different subnet sizes allow the company to assign IP addresses according to departmental needs and improve network management.

- A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

  - Convert the given IPv6 address into its compressed notation.

  - Expand	the	compressed	address	back	into	its	full	hexadecimal representation.

  - Identify the network prefix using the /64 prefix length.

  - Calculate the total number of host addresses supported within the subnet.

  - the total number of possible host addresses available within this network.

ANS: Smart Hospital – IPv6 Addressing

Given IPv6 address: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

1. Convert into compressed notation

- IPv6 addresses contain eight groups of hexadecimal numbers.

- Leading zeros in each group can be removed.

- Consecutive groups of zeros can be replaced with ::.

- Compressed notation: 2001:db8:abcd::/64

2. Expand into full notation

- The :: symbol represents consecutive zero groups.

- The full address is:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

3. Identify the network prefix

- Prefix length: /64

- Total IPv6 address length: 128 bits.

- The first 64 bits identify the network portion.

- Network prefix: 2001:0db8:abcd:0000::/64

4. Calculate the number of host bits

- Total bits: 128

- Network bits: 64

- Host bits: \(128-64=64\)

5. Calculate total addresses

- Formula: \(2^{64}\)

- Total addresses: 18,446,744,073,709,551,616

- IPv6 provides a very large number of addresses for connected devices.

Conclusion: IPv6 supports the hospital's growing number of medical devices, smart sensors, and other connected equipment. It provides a large address space for future expansion.

- An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries: 192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

  - The destination subnet to which the packet belongs.

### The matching routing table entry.

  - The next-hop network/interface used to forward the packet.

  - The default route that should be used if the destination does not match any routing table entry.

ANS: ISP Router – Routing Table

Given routing table:

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

Destination IP address: 192.168.2.45

1. Determine the destination subnet

- The destination IP address is 192.168.2.45.

- The first three octets match the network 192.168.2.0/24.

- Therefore, the destination subnet is 192.168.2.0/24.

2. Identify the matching routing entry

- Matching entry: 192.168.2.0/24

- The router checks the destination IP against its routing table.

- It selects the matching entry to forward the packet.

3. Determine the next-hop network/interface

- The router forwards the packet through the interface connected to 192.168.2.0/24, if directly connected.

- If the network is reached through another router, the configured next-hop IP address is used.

- The exact interface or next-hop IP cannot be identified because it is not provided in the question.

4. Identify the default route

- Default route: 0.0.0.0/0

- It is used when no specific routing-table entry matches the destination.

- The packet is forwarded to the configured default gateway or next-hop router.

- If no default route exists, the router may drop the packet.

Conclusion: A routing table helps the router choose the correct path for packets. This improves communication between different networks.

  - A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

    - Determine the network address and broadcast address for each LAN.

    - Identify the valid host IP address range.

    - Suggest suitable IP addresses for three computers in each LAN.

    - Identify the default gateway that hosts in each LAN should use for communication.

ANS: Administration and Finance LAN – Subnetting

Given subnet information:

- Administration LAN: 192.168.10.0/26

- Finance LAN: 192.168.10.64/26

- Administration gateway: 192.168.10.1

- Finance gateway: 192.168.10.65

1. Administration LAN

- Network address: 192.168.10.0

- Subnet mask: 255.255.255.192

- Broadcast address: 192.168.10.63

- Valid host range: 192.168.10.1 – 192.168.10.62

- Usable hosts: \(2^6-2=62\)

- The network supports computers and other devices in the administration department.

2. Finance LAN

- Network address: 192.168.10.64

- Subnet mask: 255.255.255.192

- Broadcast address: 192.168.10.127

- Valid host range: 192.168.10.65 – 192.168.10.126

- Usable hosts: \(2^6-2=62\)

- The network supports finance computers and related office devices.

3. Suitable IP addresses for three computers

Administration LAN

- Computer 1: 192.168.10.2

- Computer 2: 192.168.10.3

- Computer 3: 192.168.10.4

Finance LAN

- Computer 1: 192.168.10.66

- Computer 2: 192.168.10.67

- Computer 3: 192.168.10.68

- All these addresses belong to their respective subnets.

- They do not conflict with the configured gateway addresses.

4. Identify the default gateways

- Administration default gateway: 192.168.10.1

- Finance default gateway: 192.168.10.65

- Each computer must be configured with the gateway belonging to its own LAN.

- The router uses these gateways to forward packets to other networks.

- The Layer-3 router enables communication between the Administration and Finance LANs.

Conclusion: Correct IP addresses, subnet masks, and gateways help both departments communicate properly. Separate subnets also make network administration easier.

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understandin g of the scenario and context | Shows good understandi ng with minor gaps | Partial understandin g; misses key aspects | Misinterpret s or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganize d and difficult to understand |