# SIMATS ENGINEERING

### ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30% Software:Cisco Packet Tracer,Wireshark

#### QUESTIONS: 5	Total Marks:50

Annexure A

## Debugging & Optimisation Task

##### Code of Conduct:

I NIVEDITHA / 192421440L (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

##### Signature of the student:

##### NIVEDITHA

Annexure A

### Debugging & Optimisation Task

- A network administrator has configured a workstation with the following settings:

  - IP Address: 192.168.10.65

  - Subnet Mask: 255.255.255.192 (/26)

  - Default Gateway: 192.168.10.1

However, the workstation is unable to communicate with other systems configured in the subnet 192.168.10.0/26.

Task

Debug the network configuration by:

  - Identifying the subnet to which the configured IP address belongs.

  - Determining the network address, broadcast address, and valid host range.

  - Verifying whether the configured IP address and gateway are valid.

  - Identifying the configuration errors.

  - Optimizing the configuration by assigning an appropriate IP address and default gateway.

  - Calculating the total number of usable host addresses after correction.

ANS: Debugging IPv4 Configuration

Given:

- IP address: 192.168.10.65

- Subnet mask: 255.255.255.192 (/26)

- Default gateway: 192.168.10.1

- Required subnet: 192.168.10.0/26

1. Identify the subnet

- The IP address 192.168.10.65 belongs to 192.168.10.64/26.

- It does not belong to the required subnet 192.168.10.0/26.

2. Network address

- Required network: 192.168.10.0

3. Broadcast address

- Required subnet broadcast: 192.168.10.63

4. Valid host range

- 192.168.10.1 – 192.168.10.62

5. Check IP and gateway

- The configured IP is valid in a different subnet.

- The gateway 192.168.10.1 belongs to the required subnet.

- Therefore, the IP and gateway are in different subnets.

6. Configuration error

- The workstation has an incorrect IP address for the required subnet.

7. Correct configuration

- IP address: 192.168.10.2

- Subnet mask: 255.255.255.192

- Default gateway: 192.168.10.1

8. Usable host addresses

- Formula: \(2^6-2=62\)

- Total usable hosts: 62

Conclusion: Correcting the IP address places the workstation in the proper subnet and allows communication with other devices in that network.

- Two devices are configured with the following IPv6 addresses:

  - Device A: 2001:db8::ff00:42:8329/64

  - Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

  - Expanding both IPv6 addresses into their complete notation.

  - Determining the network prefix of each device.

  - Verifying whether both devices belong to the same subnet.

  - Identifying the addressing or prefix mismatch.

  - Optimizing the IPv6 configuration by assigning appropriate addresses.

  - Calculating the number of available host addresses within the corrected subnet.

ANS: Debugging IPv6 Configuration

Given:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

1. Expand Device A

2001:0db8:0000:0000:0000:ff00:0042:8329/64

2. Expand Device B

2001:0db8:0000:0000:0001:0000:0000:0001/64

3. Identify network prefixes

- Device A prefix: 2001:db8:0:0::/64

- Device B prefix: 2001:db8:0:0::/64

4. Check whether they belong to the same subnet

- Both devices have the same first 64 bits.

- Therefore, both belong to the same /64 subnet.

- Their addresses do not have a network-prefix mismatch.

5. Identify the possible error

- The given addresses alone do not establish the cause of communication failure.

- Check the subnet configuration, duplicate addresses, firewall rules, and device settings.

6. Optimize the configuration

- Keep both devices in the same /64 subnet.

- Ensure that each device has a unique IPv6 address.

- Verify connectivity and firewall settings.

7. Available host addresses

- Host bits: \(128-64=64\)

- Total addresses: \(2^{64}\)

- Total: 18,446,744,073,709,551,616

Conclusion: Both devices belong to the same IPv6 subnet. Other network settings should be checked to identify the communication problem.

- An organization has allocated the network 192.168.1.0/24 into several departments using the subnet masks /26, /27, and /28. Some departments report insufficient IP addresses, while others have a large number of unused addresses.

Task

Analyze and optimize the subnet allocation by:

  - Identifying the network address, broadcast address, and host range for each subnet.

  - Calculating the number of usable and unused IP addresses.

  - Identifying inefficient subnet allocations.

  - Debugging the addressing plan by locating subnet allocation mistakes.

  - Recommending an optimized subnetting scheme using VLSM to improve address utilization while supporting current departmental requirements.

ANS: IPv4 Subnet Allocation Using VLSM

Given network: 192.168.1.0/24

The organization uses /26, /27, and /28 subnets, but some departments have insufficient addresses while others have unused addresses.

Answer

1. Subnet /26

- Subnet mask: 255.255.255.192

- Total addresses: 64

- Usable hosts: 62

- Example network: 192.168.1.0/26

- Broadcast address: 192.168.1.63

- Host range: 192.168.1.1 – 192.168.1.62

2. Subnet /27

- Subnet mask: 255.255.255.224

- Total addresses: 32

- Usable hosts: 30

- Example network: 192.168.1.64/27

- Broadcast address: 192.168.1.95

- Host range: 192.168.1.65 – 192.168.1.94

3. Subnet /28

- Subnet mask: 255.255.255.240

- Total addresses: 16

- Usable hosts: 14

- Example network: 192.168.1.96/28

- Broadcast address: 192.168.1.111

- Host range: 192.168.1.97 – 192.168.1.110

4. Identify inefficient allocation

- Large subnets waste addresses when departments have few devices.

- Small subnets cannot support departments with many devices.

- Incorrect subnet boundaries can cause overlapping networks.

5. Optimize using VLSM

- VLSM means Variable Length Subnet Masking.

- Assign larger subnets to departments with more devices.

- Assign smaller subnets to departments with fewer devices.

- Allocate subnets from largest to smallest to use addresses efficiently.

6. Unused addresses

- In the example allocation above, 192.168.1.112 – 192.168.1.255 remains unallocated.

- This range can be allocated to other departments after checking their requirements.

Conclusion: VLSM reduces IP address wastage and provides flexible subnet sizes. Exact department allocations require the number of devices needed by each department.

- Two devices are configured with the following IPv6 addresses:

  - Device A: 2001:db8::ff00:42:8329/64

  - Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

  - Expanding both IPv6 addresses into their complete notation.

  - Determining the network prefix of each device.

  - Verifying whether both devices belong to the same subnet.

  - Identifying the addressing or prefix mismatch.

  - Optimizing the IPv6 configuration by assigning appropriate addresses.

  - Calculating the number of available host addresses within the corrected subnet.

ANS: Debugging IPv6 Configuration

Given:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

Answer

1. Expanded Device A

2001:0db8:0000:0000:0000:ff00:0042:8329/64

2. Expanded Device B

2001:0db8:0000:0000:0001:0000:0000:0001/64

3. Network prefix

- Device A: 2001:db8:0:0::/64

- Device B: 2001:db8:0:0::/64

4. Subnet verification

- Both devices belong to the same /64 subnet.

- There is no mismatch in their first 64 network bits.

5. Possible causes of failure

- Incorrect interface configuration.

- Duplicate IPv6 addresses.

- Firewall blocking communication.

- Incorrect routing or disabled network interfaces.

6. Corrective actions

- Verify the IPv6 address and /64 prefix on both devices.

- Ensure both interfaces are active.

- Check firewall settings and routing.

- Test connectivity using ping with IPv6.

7. Available addresses

- Host bits: 64

- Total addresses: \(2^{64}\)

- Total: 18,446,744,073,709,551,616

Conclusion: Both devices have the same network prefix. Troubleshooting should focus on their interfaces, firewall settings, and connectivity.

- A router contains the following routing table.

#### Destination Network	Next Hop

192.168.1.0/24	10.0.0.2

192.168.2.0/24	10.0.0.3

Default Route	10.0.0.1

Users report that packets destined for 192.168.2.100 are not reaching the intended network.

Task

Debug the routing configuration by:

- Identifying the matching routing entry using the longest-prefix match.

- Determining the next-hop router selected for packet forwarding.

- Verifying whether the destination IP belongs to the intended subnet.

- Identifying any routing configuration errors.

- Optimizing the routing table to improve packet forwarding efficiency.

- Identifying the route that will be used if no matching destination exists.

ANS: Debugging Router Configuration

Given routing table:

| Destination network | Next hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default route 0.0.0.0/0 | 10.0.0.1 |

Destination IP: 192.168.2.100

Answer

1. Identify the matching routing entry

- Destination IP: 192.168.2.100

- Matching network: 192.168.2.0/24

- The destination belongs to this subnet.

2. Longest-prefix match

- The router selects the most specific matching route.

- The /24 route is more specific than the default /0 route.

- Therefore, 192.168.2.0/24 is selected.

3. Determine the next-hop router

- Selected next hop: 10.0.0.3

- The router forwards the packet toward this next hop.

4. Verify the destination IP

- Valid host range: 192.168.2.1 – 192.168.2.254

- The destination 192.168.2.100 belongs to this range.

- Therefore, the destination IP is valid for the subnet.

5. Identify possible errors

- The next-hop router may be unreachable.

- The next-hop address may be incorrectly configured.

- The destination router may lack a return route.

- Firewall rules may block packets.

6. Optimize the routing table

- Verify that 10.0.0.3 is reachable.

- Confirm that the route points to the correct network.

- Check the return path and router interfaces.

- Remove incorrect or duplicate routes if present.

7. Default route

- Default network: 0.0.0.0/0

- Default next hop: 10.0.0.1

- It is used when no more specific route matches the destination.

Conclusion: The correct route for 192.168.2.100 uses next hop 10.0.0.3. Checking next-hop reachability and return routing can help resolve the communication problem.

#### RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient ) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understandin g | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justificatio n | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structured code with proper comments and documentatio n | Code is mostly clear with minor documentat ion gaps | Code lacks structure and proper documentatio n | Poorly written code with no documentatio n |